Scientific production enterprise "GeoSystem"

Digital photogrammetric station 
“Delta”

Raster triangulation software

Triada for Windows

version 3.0

User’s Manual

Vinnitsa

2005

Contents

Overview	4

Basic Agreements	4

Basic skills of navigation and measurements	4

Correlation modes	5

Main menu	5

Overview	5

«File» menu	5

«File|New» command	5

«File|Open» command	5

«File|Save» command	6

«File|Save As» command	6

«File|Load ground points» command	6

«File|Load projection centers» command	6

«File|Export measurements to» command	6

«File|Save orientations» command	7

«File|Export orientation elements» command	7

«File|Generate sketches» command	7

«File|Exit» command	7

«View» menu	7

«View|Correlation» command	7

«View|Parallaxes» command	8

«View|Tie errors» command	8

«View|Orientation errors» command	8

«View|Numbers» command	8

«View|Image points» command	8

«View|Current point only» command	9

«View|Find current point» command	9

«View/Cursor color» command	9

«ViewIPoint color» command	9

«View|Point brightness» command	9

«Edit» menu	9

«Edit|Undo» command	9

«Edit|Add» command	9

«Edit|Delete|Central photo» command	9

«Edit|Delete|All photos» command	10

«Edit|Delete|Selective» command	10

«Edit|Rename» command	10

«Edit|Find wrong points» command	10

«Edit|Go next» command	10

«Edit|Back» command	10

«Edit|Reset relative» command	10

«Tools» menu	11

«Tools|Block analyzer» command	11

«Tools|Filter» command	11

«Tools|Add points» command	11

«Tools|Ground points» command	11

«Tools|Projection centers» command	12

«Tools|Block configuration» command	12

«Tools|Modify block» command	12

«Tools|Raster images» command	13

«Tools|Options» command	13

«BlockMSG/PhotoCom» menu	14

«Check conjunctions» command	14

«Interactive adjustment» command	15

«Adjust block» command	15

«Use ground points» command	15

«Use projection centers» command	16

«Selected strips» command	16

Creating the layout of triangulation block (TrianPr.exe)	16

New block	16

Correction	17

Measurements (Triada.exe)	17

Block preparation	17

Creating a block project	17

Measuring the fiducial marks	18

Measuring the photogrammetric points	19

Remeasuring the point	20

Preliminary analyzing and blunder detection	20

Adding the ground-control points	21

Bundle adjustment	21

Sequence of bundle adjustment	21

Control of results and  blunder detection	21

# Overview

The program automatically performs triangulation measurements with further adjustment using raster photos. The measurements can be made in automatic and semiautomatic modes with control by residual parallaxes and errors on tie points. Measurement of fiducial marks is fully automatic.

The built-in module of on-line adjustment makes block adjustment and absolute orientation for all block images.

Input data for adjustment are measurements of tie points and coordinate list of ground/control points and optionally, projection centers. You can perform adjustment using control points only, projection centers only and both.

As a result of the adjustment geodetic coordinates of tie (photogrammetric) points are calculated. These data are used for absolute orientation of all raster images in automatic mode.

Overview of main menu sections

Creating the layout of triangulation block (TrianPr.exe)

Measurements (Triada.exe)

Bundle adjustment

# Basic Agreements

Here the basic agreements are used in this document:

Keywords and phrases are set off in bold.

Main menu items, button captions and other interface elements are marked with blue.

Key's captions and mouse buttons are marked with red.

Notes or remarks are italicized.

The left mouse button is meant unless otherwise is specified.

Main menu items are shown like File|Open. That means command Open from File item of main menu. Unless pop-up menu is specified, it always means the main menu.

# Basic skills of navigation and measurements

To zoom image, use <+> and <-> keys, or press and hold <Ctrl> and <Alt> keys simultaneously for navigation mode. Use navigator in the upper left corner for fast moving through the image. To move smoothly, press and hold <Shift> key, when the cursor is over central image (the marker changes its shape), press the left mouse button while holding <Shift> key and move the image to required position.

If you lost current point, press <Home> key.

Point registration (measurement) is made on central image only. The cursor has form of red, blue or green cross depending on option in View menu. After registration program automatically measures points on neighbor images.

Zoom-window in the upper part of the display shows the cursor position on the current image.

Point registration is also available in this window. For that, you can use one of three modes:

1) Press <Z>  key to move Zoom-window to the cursor zone.

2) Press and hold <Ctrl> key to freeze Zoom-window, move cursor and register point.

3) Activate AutoPopup mode by clicking the right mouse button on Zoom-window or selecting AutoPopup from popup menu. The window will automatically move to the cursor zone after it stops.

# Correlation modes

The program uses different correlation modes for automatic measurements of different points. When you register first (central) point of the image, it will use the widest area for searching the corresponding point on the right image (far range), then the point position for the right image will be adjusted. To measure another points, the program uses normal range.

If you choose near correlation range (the green button is pressed), program will use rapid search of the points.

When using the none correlation range all measured points are situated in a position specified by operator.

You can change parameters of the correlation modes in Tools|Options menu (Relative tab).

# Main menu

## Overview

There are the following sections in the main menu:

File  View  Edit  Tools  BlockMSG/PhotoCom  Help

The Help section contains information about the latest changes in the program (What is new), help file (Contents) and information about the program (About...).

Opening the About... window you can update programs via Internet, look through the activated components, order program update and new components.

## «File» menu

### «File|New» command

Use this command to create a new triangulation project. The program opens a window where you can enter parameters of the new block.

It runs the separate TrianPr program to create block scheme and set up all block parameters.

### «File|Open» command

When you use this item, the program displays standard Windows dialogue box, where you can select file name and format for triangulation block.

You can open files of such formats:

Triangulation files TMF/TBD

It is standard program format. The file of TBD (Triangulation Block Description) format is created by TrianPr program and contains initial block scheme. This is text file.

When you load the TBD-file to Triada program, TMF (Triangulation Measurements File) file will be created, where all triangulation data (measurements, coordinates of ground points and projection centers, etc.) are saved. This working file accumulates all the triangulation data for block.

PAT-B measurements DAT/TXT

This format is used to load measurements in РАТ-В format.

### «File|Save» command

This command saves triangulation block to file with the same name.

### «File|Save As» command

When you use this item, the program displays standard Windows dialogue box, where you can select file name and format for triangulation block.

You can save file as Triangulation file (TMF) in current version of the program.

### «File|Load ground points» command

This command loads the list of ground/control points' coordinates for block.

The list should have point number and then coordinates (N X Y Z) divided by spaces or tab-symbols.

If you use points without full coordinates, you should use * symbol instead of unknown values.

For example:

87411  *  *  129.28

87412  4139975.57   751851.01  *

The first point is vertical (height) ground point, the second one is horizontal (plane).

### «File|Load projection centers» command

This command loads file with the list of GPS projection centers coordinates.

The list should have point coordinates and number (N X Y Z).

The number of center must be the same as the number of photo (name of the image file).

If you use centers without some coordinates, you should enter * symbol instead of unknown values.

### «File|Export measurements to» command

If you do not use built-in adjustment engine (PhotoCom or BlockMSG), this command helps you to export data to other adjustment programs.

You can export measurements to the following formats:

РАТ-В, РАТ-М - PАТ bundle adjustment program;

ASCII - text formats.

If you have built-in adjustment engine, select it from drop-down list on Save tab of Tools|Options|Save menu. You have to specify the full path where the selected program is.

To run bundle adjustment engine use BlockMSG or PhotoCom section in the main menu.

In this case measurement and all necessary data export will be transferred to adjustment program and returned back automatically.

After adjustment the absolute orientation of all block images can be performed automatically.

### «File|Save orientations» command

You can save inner and absolute orientation results for all block images to Models.ini file and headers of raster files.

If you measure fiducial marks only, the program will save results of the inner orientation only.

If you have finished all measurements and adjusted the block, the program saves results of the inner and absolute orientation.

When you use built-in bundle adjustment engine this operation is not required. Every time you make bundle adjustment using BlockMSG (Photocom) menu, the orientation results are saved automatically (if you click OK button in window with list of adjusted points coordinates).

### «File|Export orientation elements» command

It is used for export absolute orientation elements after block adjustment to other programs.

All output formats are described in Models.fmt file and can be modified.

### «File|Generate sketches» command

Activate this command to generate sketches of photogrammetric points.

They can be used for logging/archiving purpose or for absolute orientation of the images on analytical stereoplotters.

You can select file format for sketches in Tools|Options|Save menu.

### «File|Exit» command

This command closes the program.

## «View» menu

### «View|Correlation» command

Use this command to display correlation coefficient for every point.

This value displays how much the image on the central photo coincides with the image on the current photo. It can be in range from -1 to 1.

Generally the coefficient value should be more than 0.7, but in some cases it can be less.

The correlation coefficient is not an absolute criterion of measurement accuracy.

Residual parallaxes and tie point errors give more reliable information about real accuracy of your measurements.

Depending on the correlation value it will be highlighted with different color: green (good correlation) 0.8-1.0, yellow (normal) 0.6-0.8 and red (poor) <0,6.

### «View|Parallaxes» command

Use this command to display the parallax value for every point.

The measurement accuracy is shown by the value of residual parallax. Usually this value should not exceed half a pixel size.

Depending on the parallax value the point will be highlighted with different color: green (good accuracy) - less than 1/2 pixel, yellow (normal) 1/2-1 pixel and red (poor) more than 1 pixel.

You can activate this mode simultaneously with correlation mode.

In such case a point color will be defined by its parallax.

### «View|Tie errors» command

Tie error is the difference of 3D-point coordinates in relative reference system of two neighbor models.

It is shown in meters and can be calculated for points in central column of every image (triple overlap zone).

These errors calculated approximately and depend on average flight scale (it is set when you create a new triangulation block). You can change the average scale and other parameters in Tools|Block configuration menu.

If the scale is not correct, the calculated errors will be incorrect as well.

### «View|Orientation errors» command

Orientation errors can be shown after you have done bundle adjustment and images orientation.

The orientation error is the difference between geodetic coordinates of a point after bundle adjustment and geodetic coordinates calculated from measured point position on left and right image using absolute orientation elements of these images.

It is displayed in meters in the right column of block analyzer and on the images as red arrow.

The arrow on every photo begins at current measured position of a point and goes to position that calculated from its geodetic coordinates using absolute orientation elements. This vector can be a hint that helps to correct positions of wrong points and then repeat adjustment.

### «View|Numbers» command

This command shows/hides point numbers on the images.

### «View|Image points» command

If you use DIPEdit program from Delta software package, you can mark points on different images (e.g. ground points for triangulation block).

The number and position of such point will be written to the header of the raster file. You can switch on/off display of these points.

The points are shown for your reference only and do not included to triangulation block unless you measure them in Triada program to add to the project.

### «View|Current point only» command

Use this command to view only current point on the images.

### «View|Find current point» command

Use this command to return to current point. You can also use <Home> key.

### «View/Cursor color» command

Сursor color can be changed by this command.

The cursor on the central image is cross-shaped with changeable color to provide the best visibility.

### «ViewIPoint color» command

Use this command to change color scheme for points on the images to provide the best visibility.

### «View|Point brightness» command

Use this command to decrease point brightness on light images.

## «Edit» menu

### «Edit|Undo» command

It cancels last editing operation.

The program has a log of operations performed with a block. This allows canceling them in the reverse order.

### «Edit|Add» command

The program adds a tie or a ground point.

You should set point number in the appeared window.

After you have finished measurement, it is enough to click left mouse button on any image to add a new point.

### «Edit|Delete|Central photo» command

Use it to delete point measurement on the central (active) image.

The image where mouse cursor is cross-shaped is the active one.

### «Edit|Delete|All photos» command

This command deletes selected point measurements on all images.

### «Edit|Delete|Selective» command

This command deletes point measurements on any selected images.

It is necessary when the point does not locate in triple overlap zone on the current and lower strips.

In such case the point is measured on 5 images only and should be deleted from one of 6 images.

To restore point measurement it is enough to drag it from central (active) photo to required place on the image where it is absent.

### «Edit|Rename» command

You can change point number on all the block images.

### «Edit|Find wrong points» command

The program performs search of wrong points in three stages:

1) Non-measured points (their correlation value equals to 0).

2) Big values of the parallax (>30 micrometers).

3) Big value of tie errors (>60 micrometers).

### «Edit|Go next» command

The program goes to next point.

The parameters for automatic moving to next point can be set in Tools|Options|Relative menu.

### «Edit|Back» command

This command allows you to check and re-measure previous points.

### «Edit|Reset relative» command

Use this function to start measurements from the very beginning.

Be careful - it deletes measurements of all tie and ground points. The program will return to the results of inner orientation (after measurement of fiducial marks).

## «Tools» menu

### «Tools|Block analyzer» command

The Block analyzer is used to check point measurements, locate blunders and fix them. Such control can be done using data of inner and relative orientation (single model control), tie errors (along strip control) and bundle adjustment data (total control).

The program opens window where the point number, results of inner and relative orientation and correlation coefficient are displayed. The point, which value considerably exceeds mean-square value, are highlighted.

The results of inner orientation are displayed for each image in block. The results of relative orientation are displayed for all photos except the first image of every strip. These values are shown in microns.

The tie point errors are displayed for all photos except the first and the last image in strip. These values are measured in meters.

If View|Orientation errors option is on, then these errors will be displayed in the block analyzer instead of correlation coefficient (most right column).

To correct a point measurements locate it in Analyzer and then use mouse click on an image to move it to new position.

When you change point position on central (active) image it will be automatically changed also on other images (using correlation).

Parallaxes and tie errors will be recalculated. To cancel the change use Edit|Undo command.

### «Tools|Filter» command

You can automatically filter points by values of their correlation, parallaxes and tie errors.

Define how many points in each zone you want to filter by each criterion.

This function is usually used for automatic measurements where number of points in zone can be very big.

### «Tools|Add points» command

This command automatically adds point measurements by their geodetic coordinates. It is usually used to add ground points automatically and then specify their exact positions. The program will display the list of all points which coordinates are in the list of ground points for the block (Tools|Ground points).

Then the program adds points with accuracy of preliminary orientation. To specify their exact position use block analyzer.

Use Unmeasured button to measure next point from the added ones.

This command requires preliminarily (rough) adjustment. To do that, locate and measure 4-5 ground points manually and perform bundle adjustment and images orientation. As the result will be calculated absolute orientation elements for all images that allows for program automatic location of points by their geodetic coordinates.

### «Tools|Ground points» command

The program opens a list of ground points for the block.

To load ground points use File|Load ground points command.

Once loaded the points are saved to TMF-file for future using and can be shown and edited by this command.

The last column in the list defines point status:

1 - horizontal ground point

2 - vertical ground point

3 - ground point with all coordinates

0 - point obtained as a result of bundle adjustment.

The status is assigned automatically when you load the coordinate list.

Anyway later you can change it on your own.

You should add K symbol to the point status if you want to make it control point. To do that use double click on point status column.

Control points are not used as ground in adjustment, and their adjusted coordinates are calculated.

Then you can see the difference between adjusted and real coordinates in the log to estimate and check block accuracy.

### «Tools|Projection centers» command

The command opens a list of the projection centers of the images obtained from GPS.

To load a list of center coordinates use File|Load projection centers command.

Once loaded the centers are saved to TMF-file for future using and can be shown and edited by this command.

### «Tools|Block configuration» command

This command opens a window where you can change parameters of the triangulation block (average scale, end and side overlap, etc) and select a camera for each strip of the block. These values come from initial block description (TBD-file) and can be changed at any time.

Average scale is very important for tie points errors shown in Block Analyzer. Please be sure that you set up here correct value.

### «Tools|Modify block» command

This command allows you to change block structure by adding or deleting the images (strips).

There are the following groups you can change:

Insert photos (Right) - inserts images on the right of the current one;

Insert photos (Left) - inserts images on the left of the current one;

Delete photos - deletes photos from the strip;

Add strips - adds strips to the triangulation block.

You should enter file name with tmf-extension to add its strips to the end of the block.

The block with large number of the strips is often divided into parts to be measured by different operators.

After the strip is added you should add tie points between the strips (strip overlap points).

Delete strips - deletes strips from the triangulation block.

Shift strip to right - shifts strip on the specified number of the images to the right.

Shift strip to left - shifts strip on the specified number of the images to the left.

### «Tools|Raster images» command

This command displays list of raster files included to the current block.

It is often used to change the path to images when you transfer your project to another computer.

### «Tools|Options» command

This command opens the dialog box to change the program settings.

It consists of pages (tabs) grouped by functions.

Click OK to save changes you have made.

General

Interface language

You can set language of program's menus, dialogs and messages in this drop-down list.

Automatic	The program automatically defines interface language by Windows version.

English	Sets English interface language.

Russian		Sets Russian interface language.

Restart the program to activate these changes.

Stereo interface

You should select device you are going to use for work in stereomode.

Stereoscope - an operator uses stereoscope.

Glasses (V3800) - an operator uses shutter stereoglasses with ASUS V3800 videocard.

Glasses (V6800)  - an operator uses shutter stereoglasses with ASUS V6800 videocard.

Glasses (OpenGL) - an operator uses shutter stereoglasses with OpenGL videocard.

Anaglyph - an operator uses anaglyph glasses.

Relative

Here you set correlation parameters for tie points measurement.

Far correlation (Red)

You can set values of gross and net zoom, aperture and search range. The mode of far correlation is used to measure first (central) point of every image and tie points between strips (points in down row).

Normal correlation (Yellow)

Here the parameters of normal correlation mode can be set. It is used to measure the most of points.

Near correlation (Green)

The near mode is used when you press and hold <Alt> key during point registration or dragging.

The mode of current point measurement depends on specified correlation mode.

Go next if correlation better than

Enter correlation coefficient to go to the next point if its value is better than specified one. It is 0,7 as default.

Reset

The program resets your settings and uses default values.

Save

There are settings of the following parameters on this tab:

PAT-B output format

Sets a format of measurement export to РАТ-В program. It is recommended to set New format for the latest program versions.

Export to...

Select program for export of measurements. Do not use this item if you work with built-in module for bundle adjustment (BlockMSG or PhotoCom). Data export to bundle adjustment program is independent of this setting.

Generate sketches as...

Sketches are used for data export to analytical instruments (absolute orientation) and for measurement of tie points between strips. They are generated in \TRIADA\Sketches directory.

You can select Bitmap, JPEG or HTML/JPEG format for their generation. The most convenient format is HTML-format.

Minimize - check on this option to minimize program while generating sketches.

Clean folder - deletes all files from Sketches folder before sketch generation.

Swap XY - swaps coordinates on the sketches, if you work with UTM-system.

Bundle adjustment engine

Use this item to select bundle adjustment engine and specify its location.

Ground points accuracy, m - the bundle adjustment engine uses this option to minimize measurement errors.

Projection centers accuracy, m - the bundle adjustment engine uses this option to minimize measurement errors.

Parallax tolerance, microns - the bundle adjustment engine uses this option to check and exclude measurement errors.

Adjustment residuals visualisation scale - adjustment errors, performed in Check conjunctions item, are displayed as blue vectors on the photos.

Set different from 1 scale value to increase size of these vectors.

Compute GPS displacement (require BlockMSG 4.0) - if this checkbox is checked on, the program will automatically calculate errors in measurements of projection centers due to the displacement of GPS-receiver antenna.

After you have specified bundle adjustment engine, a new menu item for bundle adjustment will appear in the main menu.

## «BlockMSG/PhotoCom» menu

### «Check conjunctions» command

The command executes bundle adjustment engine in the mode of free network construction. . It is used as the first stage to check block measurements and integrity.

At first we recommend to check conjunctions without ground points and projection centers (uncheck Use ground points and Use projection centers menu items).

Then the program opens block analyzer with a list of points and adjustment errors.

The window consists of three tables. The first table (All points) contains a list of all points for a block:

Point	Point number

DCol	Collinearity error (in microns)

DХY	Tie error  in meters (sum vector by ХY-coordinates)

DZ	Tie error of Z-coordinate in meters

Collinearity error approximately corresponds to residual parallaxes. It is calculated for all photos where current point is.

Tie error calculated in bundle adjustment engine also takes into account point position on all photos of all strips where current point is.

The second table (Photos) contains a list of block photos with the following columns:

Photo	Photo number

Inn		Mean square residuals by fiducial marks in microns (inner orientation)

Rel		Mean square residual parallax in microns (relative orientation)

Tie		Mean square error of tie points in meters (sum vector by XYZ)

K		Mean square value of correlation coefficient

The third table (Points) contains a list of points for current photo. There are the following columns in it:

Point	Point number

PX		Residual for X-coordinate in microns (for fiducial marks only)

PY 		Residual parallax for Y-coordinate in microns

Tie		Tie error in meters (sum vector by XYZ)

DX		Tie error for X-coordinate in meters

DY		Tie error for Y-coordinate in meters

DZ		Tie error for Z-coordinate in meters

K		Value of correlation coefficient on a point

If View|Orientation errors option is activated, the last column will display value of orientation error instead of correlation. This requires preliminary bundle adjustment.

### «Interactive adjustment» command

This command opens bundle adjustment engine BlockMSG or PhotoCom in dialog mode and transfer there all the required data (measurements, coordinates of ground and control points, etc.).

After this is done, the program displays a window with results of processing the block strips and its scheme.

Detailed description of work of bundle adjustment engine can be found in PhotoCom or BlockMSG manuals.

When you exit bundle adjustment engine, the program will display a window with a list of geodetic coordinates obtained after bundle adjustment. Click ОK button in this window to perform inner and absolute orientation of all block photos.

### «Adjust block» command

This command do the same as Interactive adjustment, but starts bundle adjustment program in hidden (batch) mode. The results of interactive adjustment and batch adjustment are the same. Use interactive adjustment on the first stage to check and analyze the results. Use Adjust block command for fast repeated adjustment after some slight changes in measurements.

When adjustment is finished, the program displays a window with a list of geodetic coordinates obtained after bundle adjustment. Click ОK button in this window to perform inner and absolute orientation of all block photos.

Then the program displays a window, where you can find mean square residual for every photo and total residual for triangulation block.

If you perform image orientation, all photos are ready for mapping in mono or stereo mode in Ged program. The elements of inner and absolute orientation for each photo are saved to Models.ini file and titles of raster images.

### «Use ground points» command

Use this option to adjust block or check conjunctions taking into account block ground points.

This option effects on Check conjunctions, Interactive adjustment and Adjust block commands.

The option is ignored if there were no ground points loaded.

Ground points and projection centers are not required for preliminary measurement control. In such case the program will create free block model.

### «Use projection centers» command

Use this option to adjust block (check conjunctions) taking into account projection centers.

You can use both coordinates of projection centers and ground points or one of these data to adjust the block.

This option effects on Check conjunctions, Interactive adjustment and Adjust block commands.

The option is ignored if there were no projection centers loaded.

Ground points and projection centers are not required for preliminary measurement control. In such case the program will create free block model.

### «Selected strips» command

Use this command to adjust selected strips of a block.

Enter number of first and last strip to be adjusted in the appeared window.

Usually this command is used for preliminary adjustment when the measurement of a whole block has not completed or to check some particular strips.

# Creating the layout of triangulation block (TrianPr.exe)

## New block

To create new block of photos you should use File|New command or press corresponding button on the toolbar. The program opens a form with following parameters of a block:

Block name - text name of a block.

Average scale - denominator of the flight scale.

Average end overlap, % - it equals 60 % by default.

Average side overlap, % - it equals 30 % by default.

Flight data - text information.

Camera - it is selected from existing list of cameras.

Film - Film ID, any text information.

Average flight height above ground, m - it is an optional parameter.

Average terrain height, m - it is an optional parameter.

Strip Count - total quantity of strips in your block.

Maximal strip capacity - the maximum number of photos in strip.

After you enter block parameters, it is created by standard layout displayed in the strip table. Each photo of a block has standard file name which corresponds to photo number.

You can set parameters for each strip changing information of First name, Last name, Step rows.

This layout can be canceled by pressing Clear strip table button.

Press OK button to save new triangulation block as Noname.tbd file and to close form for entering block parameters.

Press Cancel button to ignore information about new strip.

You need to fill a list of paths for searching images and their layout.

## Correction

The parameters of a block can be viewed and changed using options from main window or additional form for entering the parameters of a block.

You can change block capacity, path for searching images and layout of the images.

Note: When you decrease block capacity, you can lose information about photos of deleting strips.

The whole block will be formatted by standard scheme and all the information will be lost if you change maximum capacity of a strip.

It is better to use editing window Photos of the program to chance block capacity.

# Measurements (Triada.exe)

## Block preparation

To prepare a layout of images/strips press Block button on Models program toolbar.

As a result TBD-file with block description will be created. It is a text file which can be viewed and edited with any text editor.

## Creating a block project

Set as current a camera which was used for survey.

Press Triangulation button on Models.exe program toolbar. It will load TRIADA.EXE program. Then use File|Open program to open TBD-file with block description.

The program creates binary file with the same name and tmf-extension. It is used for saving the block project and all measurements.

## Measuring the fiducial marks

After you open a file, the program automatically goes to first fiducial mark on the first photo of the first strip. These marks are displayed as FM-#, where # - mark number in an order specified in camera description.

Point cursor at fiducial mark and register it with left mouse button. The program will display a dialog for automatic measurement of this mark for whole block.

Select Apply to whole block item and press ОК button. The program will automatically measure first fiducial mark on all photos of the block. The measurement is done by correlation method. Correlation coefficient defines identity of fragments. If its value is less than 0.7 for one of measured photos, the program will move to this photo and allow to remeasure fiducial mark.

After you have measured it, press Cancel button in the appeared box of automatic measurement.

If all automatic measurements have correlation 0.7 and better, the program will go to the next mark. The current point is specified in the title of "mode window" where there are buttons to go to the next point (Go|Next) and to measure previous point once again (Back).

The buttons of correlation modes can be found in the same window.

## Measuring the photogrammetric points

After you have measured the last fiducial mark, the program will automatically start measurements of photogrammetric points. If it does not happen, press Go button in the "mode window".

The program displays a dialog for selecting the mode of the measurements:

Set point number in a zone (usually 1-3) and manual point selection. After that the program will create photogrammetric points on each image and go to the first (central) point on the first photo of the first strip.

Point numbers are generated automatically in the following format SS-ZNN, where SS – photo number, where a point is in the central zone, Z - zone number (0 – central, 1 – upper (top), 2 – lower (bottom)), NN - point number in the zone (00 – for the first point, 01 - for the second one, etc.).

So each photo has points located in its central zone, points from previous photo located in the left part and points of next photo of the strip.

For example: number 62-000 corresponds to the central zone of photo 62, number 61-200 corresponds to the first point of upper zone of photo 61 (it is located in the lower zone in the right part of photo 62).

The measurements are done in numbering scheme. You should measure points of the central zone of each photo. The program automatically exports them to the left and right images and images of lower strip if it is required.

Select well-defined contour and register point by clicking left mouse button. The program automatically detects this point on neighbor photos.

If minimum (worst) correlation coefficient is more than 0.7 for all measured images, the program will go to the next point (upper zone).

Register point in the same way. To remeasure previous point use Back button in the "mode window". You can also press <left arrow> and <right arrow> key.

## Remeasuring the point

If the program has not found required point on neighbor photos, you should register it once again while pressing hold <Ctrl> key. To specify point position on a proper photo, drag it from central photo (press and hold left mouse button).

When you remeasure point, you can hold pressed <Alt> key for point registration to switch to near correlation mode.

If point position is correct on separate photos, you should specify its approximate location on remaining photos. For that click left mouse button on central image (as for registration), hold it and move cursor to the required photo. Then release mouse button in a position corresponding to point location.

If you specify point position with high accuracy and want to disable correlation temporarily, you should hold pressed <Alt> key.

Repeat this operation for all photos, where you are not satisfied with point location.

Press Go button to measure next point.

## Preliminary analyzing and blunder detection

Use Tools|Block analyzer command to analyze measurements:

Block analyzer is a window with two tables. The first table (Photos) contains a list of block images and columns with the following information:

Photo	Number of photo

Inn		Mean square residual error on fiducial marks in microns (inner orientation)

Rel		Mean square residual parallax in microns (relative orientation)

Tie		Mean square error of tie points in meters (sum vector XYZ)

K		Mean square value of correlation coefficient

The second table (Pоints) contains a list of points of current photo and columns with the following information:

Point	Number of point

PX		Residual for X-coordinate in microns (for fiducial marks only)

PY 		Residual parallax for Y-coordinate in microns

Tie		Tie error in meters (sum vector by XYZ)

DX		Tie error for X-coordinate in meters

DY		Tie error for Y-coordinate in meters

DZ		Tie error for Z-coordinate in meters

K		Value of correlation coefficient on a point

The worst results are highlighted with dark color to simplify their search.

If you point cursor on a point number, the program shows its position on the current and neighbor photos. To remeasure you should click left mouse button on a new place.

You can do it on central image only.

If point position is satisfactory on all images, you can remeasure point on one of photos dragging it from central point. Press and hold <Alt> key if point position is quite accurate.

## Adding the ground-control points

After you have finished measurements, you can work in two modes.

One of them allows you to remeasure a point selected from the list in block analyzer (when block analyzer is loaded).

The second mode adds new points with arbitrary numbers - ground-control points. Move to the required image and register new point clicking the left mouse button. The program asks number and adds it on the central and neighbor photos. To edit this point you should load block analyzer and remeasure it.

# Bundle adjustment

## Sequence of bundle adjustment

You can adjust triangulation block using built-in bundle adjustment engine BlockMSG or PhotoCom (on-line triangulation).

This engine allows you to use coordinates of projection centers, adjust blocks with complicated configuration, etc. There are no special requirements to the position and order of photogrammetric and ground points in a block.

Bundle adjustment consists of three stages:

1. Checking conjunctions

2. Interactive adjustment

3. Block adjustment

The first stage is used for control of your measurements and error search. It is recommended to check conjunctions without coordinates of ground points and projection centers.

Use BlockMSG|Use ground points and BlockMSG|Use projection centers commands to enable/disable ground points and projection centers when adjusting the block.

Other stages perform adjustment and orientation. As a result you will have a list of coordinates of photogrammetric points. These points are used for absolute orientation of the whole block. To start orientation process press ОК button in the window with coordinates. Press Cancel button to interrupt loading the coordinates and cancel absolute orientation.

It is recommended to use Interactive adjustment item for analyze and search of errors.

You can use Adjust block item for fast readjustments after some changes.

They both generate a list of coordinates as a result of adjustment.

You can control adjustment results by enabling orientation errors option and using block analyzer. Orientation errors for the photo and each point on it are displayed in the right column of the analyzer and on the photo as a red arrow.

## Control of results and  blunder detection

Block analyzer allows to find errors of inner and relative orientation, tie errors, etc. You can do it before bundle adjustment because you don't need to have coordinates of ground points and projection centers. But measurements of neighbor photos and stereo models should be matched.

You should use bundle adjustment for final control of measurements. Check conjunctions command is used for control of your measurements and error search. It is recommended to check conjunctions without coordinates of ground points and projection centers.

Use BlockMSG|Use ground points and BlockMSG|Use projection centers commands to enable/disable ground points and projection centers when adjusting the block.

Results of control of tie errors are displayed in the additional list of block analyzer window.

It is recommended to use Interactive adjustment item for analyze and search of errors.

You can use Adjust block item for fast readjustments after some changes.

They both generate a list of coordinates as a result of adjustment.

You can control adjustment results by enabling orientation errors option and using block analyzer. Orientation errors for the photo and each point on it are displayed in the right column of the analyzer and on the photo as a red arrow.

After you have detected blunders, you can repeat bundle adjustment in automatic or interactive modes.