# 1. Introduction

## 1.1. Software components

PHOTOCOM software allows you to create and adjust a three-dimensioanal analytical phototriangulation using existing data, which is collected using the Digital Photogrammetric Workstation (DPW). It contains the following parts:

- Phototriangulation adjusting software

- Software for creation of the geodetic projection library

The main advantage of this software is a possibility to determine and correct the input data errors on-the-fly. This technology raises productivity and reliability of the adjustment process of the geodetic base.

Remark: All the DPW software components must be placed in the same folder.

## 1.2. Bundle adjustment software

The main module of the PHOTOCOM software is a Photocom.exe. This module has the same copyright protection as the main DPW software. Thus if you do not have a hardlock key or it was not programmed for using this software you will not receive adjusted coordinates and orientation elements report for single photos and stereopairs.

You must also have rights to use Triada.exe. Otherwise Photocom.exe works in demo mode: restricted block size and some functions are disabled.

To obtain corresponding rights contact the software developer.

To start Photocom.exe use the Triada.exe main menu item with the corresponding caption (PhotoCom). To enable it use Triada|Tools|Option|Save dialog to set the Photocom as the default bundle adjustment software. Select PhotoCom from the drop-down list of the "Bundle adjustment engine" section, then specify the path to the Photocom.exe and set corresponding accuracy parameters. A new main menu item "PhotoCom" will appear.

## 1.3. Creation of the geodetic projection library software

This software is useful for transforming adjusted coordinates from one map projection to another. Executable module is ConfProj.exe.

It must be placed into the same folder with Photocom.exe.

This program could not be used without the main component Photocom.exe. Thus it is not protected with the hardlock key.

## 1.4. Online help

Online help of the Photocom software consists of two parts:

- Photogrammetric tranguation software

- Software for creating of the geodetic projection library

## 1.5. Software developer

For the additional information concerning the Photocom software contact the developer:

Antipov Ivan, Professor of the Siberian State Geodetic Academy, Doctor of Techical Science.

Novosibirsk, Russia

Tel. +7 3502 11-45-09

e-mail: antipov@online.nsk.su

# 2. Photogrammetric triangulation

## 2.1. Main information about software

### 2.1.1. Application

Photocom.exe takes data collected using the Digital Photogrammetric Workstation (DPW) and builds strip or block three-dimentional analitical phototriangulation network.

The main DPW software provides photogrammetric measurements and phototriangulation data collection. During construction of the phototriangulation network over this data Photocom makes independent check of technology requirements compilance to the structure and accuracy of the information. This test helps DPW operator to make a decision concerning collected data correction or going to the further technological operations.

See also "Funtions of the bundle adjustment software"

### 2.1.2. Functions

Photocom software has the following functions:

- import and analysis of the existing triangulation data collected using DPW;

- building arbitrary oriented strip three-dimentional analitical phototriangulation networks with accuracy estimation of all interior photogrammetric links existing in the strip network;

- exterior orientation of the arbitrary oriented strip networks using ground points and joint adjustment of results of all photogrammetric, geodetic and airborne measurements of separate strips;

- joining of the arbitrary oriented strip networks, which have intersection zones, into an arbitrary oriented block network, joint adjustment of all photgrammetric data of this network with the analysis of identification quality of tie points of adjacent strips.

- joining of the arbitrary oriented or adjusted strip networks into the block network, exterior orientation of this network and joint adjusting of all its photogrammetric, geodetic and airborne data with accuracy estimation of point coordinates of the adjusted network;

- creating of the list of point coordinates of adjusted network;

- creating of reports of data processing for DPW operator including general characteristic of separate stages of calculation process and photogrammetric network;

- creating of a binary file for the main DPW software purposes containing estimation values, which are characterise the keeping of the main geometric rules of the network in order to display them on the screen;

- creating of a binary file containing recommendations concerning collected data correction for the main DPW software purposes.

### 2.1.3. Phototriangulation network parameters

Default projection is a transverse cylindrical projection on the Krasovsky reference ellipsiod. Ground points may be specified both in coordinate systems of 1942 or 1995 year. It is not necessary to enter ordinates with the zone number. If zone number not specified, the whole block must be located in the same coordinate zone. It is also possible to use a local coordinate system with the begining at an arbitrary point.

To use another cordinate system for ground points use ConfProj.exe module. A conformal geodetic projection file ...\Library\ProjFile must contain information about at least one projection. Do not forget to select the default projection from ...\Library\ProjFile file, if it contain several projections.

There are not limitations for focal length, photo dimensions and flight height. Software do not require map scale and accuracy values.

All photos of a strip must be photographed using the same camera. It is not allowed to use more than three cameras per block. The strip directions are arbitrary. It is possible to use main, wireframe or joint strips. There is the only condition for a strip layout in the block: strips must be linked with tie points. Software provides proceeding of any block with geometric layout of ground and determines points which meet the main photogrammetric theory requirements. Accuracy of adjustment results depends on network parameters.

Maximum strip number is 255, and maximum number of photos in a strip is 255.

## 2.2. Input data

### 2.2.1. Input data composition

All necessary information transfers from the main DPW software (Triada) to Photocom. Input data consists of several files which number depends on existing ground points and their type. At least a block project must consists of the following three files:

- phototriangulation project file *.prj, which contain input/output links and some other common for all strips information;

- photogrametric measurements file *.mea, which contain point numbers and coordinates;

- network configuration file *.use, which contains block strip list and proceeding method.

If the block has ground points, they will be stored in a ground points file *.gnd, containing numbers and coordinates of these points. It also may contain information about accuracy of the point coordinates.

In the case of existing image linear exterior elements determined using GPS, input data contains coordinates of projection centers file *.cpr. It consists of photo numbers and their projection center coordinates. It also may contain information about the accuracy of coordinates.

## 2.3. Using software

### 2.3.1. Processing modes of the software

Two processing modes of the software are the following:

1. Batch mode. In this mode neither calculations nor results are shown, excluding emergency cases. Software does not keep minutes. As a result software generate *.res and *.log files for the main DPW software purposes.

2. Interactive mode. Software shows steps of the calculation process and log file which is useful for operator. All the results are shown on the screen as tables and graphs.

Both batch or interactive mode provide you with the same accuracy results.

### 2.3.2. Executing software

The easiest way to run program is to do this from the Triada main menu. You must set corresponding options first. Use "Tools|Options...|Triada options and preferences" dialog box for this. Switch to the Save page and set Bundle adjusment engine to PhotoBlock and path to the executable file, for example:

D:\Delta32\Photocom.exe

This will add the PhotoCom item to Triada main menu.

To perform block adjustment in the batch mode use PhotoCom|Adjust block command. All measurements must be performed and ground points must be loaded using File|Load Ground Points... command. In the case of existing coordinates of projection centers load corresponding file using File|Load projection Centers command. Do not forget to set corresponding items from PhotoCom menu: Use Ground Points and Use Projection Centers.

Use PhotoCom|Interactive adjustment... command to run Photocom in the interactive mode.

To check conjunctions without adjusting use PhotoCom|Check conjunctions command.

### 2.3.3. Main window

While in dialog mode you can see the calculation process steps.

The main window of the adjusting software contains "Block phototriangulation adjustment" panel at the top. Below this panel you can see the calculation scheme. It consist of several ellipses which mean separate software modules. Green color of the ellipse means an active module, modules that have finished proceeding signed with blue color and yellow color mean modules that have not run yet. Some ellipses linked with lines. This lines show the next module to run. Depending on the input data some modules may be unclaimed and will remain yellow. There is an active module name below the scheme and operation currently executed by this module above the scheme.

During the adjusting process main window shows the cell matrix structure of the normal equation. But it is the triangle part of the matrix which does not include ground point coordinate corrections. Depending on the block size each cell of the leading diagonal correspond to one photo and each side cell means the links between a pair of photos. For the large block each cell corresponds to the block piece of 2x2 images or 3x3 images etc. All the cells available at the normal equation are coloured. During the calculation process the cells change their color. During the solving of normal equation fill color of the cells changes in top-down direction. During the invertion of normal equation matrix cell fill color changes in down-top direction.

There is the status line at the bottom of the main window. It shows a message if a mistake in input data were found. The message has red background for the alarm situations.

## 2.4. Dialog boxes

### 2.4.1. Working with the dialog boxes

While in the dialog mode all adjustment results are shown in special dialog boxes as text or graphics. They help user to analyze the information of the dialogs. There are some buttons for switching between dialog boxes. There are text captions or simple images on these buttons which describe the meaning of the buttons. Use Tab key to switch the focus between buttons. Use Enter key or click on it with the left mouse button to execute an action corresponding to the button of the dialog box.

Typical button captions and their meaning are the following:

OK  -  accept the shown information and follow to the next calculation stage

Cancel  -  cancel the calculation and close the application

Help  -  see the context help

Ignore  -  ignore calculation results of the current strip and calculate the next strip

Retry  -  change the requirements and readjust the strip

Next  -  switch to the next dialog box

Back -  switch to the previous dialog box

Text (BlockText, DifText etc.)  -  open a dialog box with the textual data

Graph (StripGraph, BlockGraph, DifGraph etc.)  -  open a dialog box with the graphical data

Photos  -  show graphical schemes of images with network points and errors on them

Yes  -  positive answer to the PC request

No  -  negative answer to the PC request

Abort  -  accept a decision of the software to abort adjusting

In addition to the buttons shown below dialog boxes may contain other control items, as well as different graphical objects for data input or selection.

Most dialog boxes could be resized. Use corresponding buttons at the right-top corner of the dialog box or drag the edge of the dialog box to resize it. It is possible to zoom in the graphical images using this  button. Use this  button to zoom out. This  button restores an original image scale.

Graphical dialogs allow you to create captions of point numbers or show error values as a textual notes or vectors. Use "Number", "dX, m", "Vectors dXY" buttons and so on to see the corresponding information. To see all the information about the point at once click with the left mouse button on this point. If the strip axis is shown on the scheme it is possible to view a photo number clicking with the right mouse button on the corresponding spot of the strip axis.

It is possible to switch between photos in image scheme dialog boxes changing their numbers in the corresponding object. Using the same way you can see any photo on the dialog box of the block strip specifying its number or index.

You can display errors on the graphical dialogs starting from the particular absolute value. Use the corresponding objects to set the error limits.

You can also meet other objects which are not listed above. You could understand their meaning from experience.

## 2.5. Results of the strip compilation

### 2.5.1. Overview

In the dialog mode file StripInd.bbb containing the results of the strips compilation is displayed as the "Results of the strip compilation" dialog window. This window contain several pages which correspond to different stages of the calculation process. Some of them might be omitted depending on the calculation task.

See chapters below for more information about the contents of this protocol.

### 2.5.2. SECTION SM0. General information on the strip

This section describes the block name, strip ID, relative photography height, focal length of the camera, average errors of the parllax and image coordinate measurements and number of stereopairs in the strip.

### 2.5.3. SECTION SM1. Points distribution in the strip

This section combines the image list for the images with rearranged transfer points and the table M4 "Number of points on separate images and stereopairs". Software made an point rearrangement if  photogrammetric measurements file have a difference in the transfer point sequence for the previous and the next stereopair. Thus, the point sequence in the output file may not correspond to the point sequence in the input file.

### 2.5.4. SECTION SM2. Preliminary strip compilation

There are tables which characterize a stage of elementary strip units forming and their combination into the phototriangulation network in this section. Pay attention to table M7 "The list of points with large Y-parallaxes (Q)". Condemned during the relative orientation points signed with asterisk symbol (*). It is necessary to repeat measures of these points or exclude and replace them with new ones.

Remark. Software makes point condemnation for the cases of redundand data. If a standard zone has only one point, it could not be condemned even for the intolarable residual vertical parralax on this point.

Table M8 "Realtive orientation accuracy" describes the relative orientation results of stereopairs of the strip. In the worse case you will see the warning "Not oriented" which means unsatisfactory quality and composition of measurements to orient stereopair. If mean and root mean square values of residual vertical parralaxes are 2-3 times as small as the pixel size and maximum values not exceed pixel size it means a sufficient accuracy of the relative orientation.

Nevetheless it is necessary to analyse table M9 "Quality estimation of the relative orientation scheme". Quality stands for relation between point coordinate errors of the extracted elementary model on the stereopair and photogrammetric measurements accuracy. Ideal quality value is equal to 1.0. Values about 2.0 mean the large deformation of the elementary unit and possible strip rupture at this point. The cause may be hidden in bad geometrical layout of the stereopair points, considerable deviation from the standard layout, insufficiency of the control points at stereopair corners. This values are not admitted and the measurements of the corresponding stereopair must be performed.

The M10 table allows to estimate the geometric layout of the relative orientation via superfluity coefficients of the measurements. They describe the influence on the residual transverse parralax of the photogrammetric measurments errors on the particular point (diagonal coefficient) and of the other points of the pair (side coefficient). The ideal diagonal coefficient is equal to 1.0 and the ideal side value is equal to zero. It is impossible to find a point which has been inaccurately measured for the values of both coefficients nearly 0.5 using the residual transverse parralax values. It is necessary to place the points on stereopair as a group of 2-3 points to provide a good geometric layout.

The arbitrary oriented strip model is created relatively to the first photo in the strip. Necessary relative incline angles of photos are determined from the relative orientation. Photography bases of the separate elementary units are reduced to the same scale by the projecting ray lengths of the tie points of adjacent stereopairs. Software matches common projection centers of neighbour units and  calculates coordinate deviations on the transfer points, which have been determined for the different pairs.

This process is coming along with tabulation. The first table M11 named "List of tie points which have large coordinate deviations on adjacent pairs" describes rough mistakes at a stage of selection and measurements of the transfer points. Condemned points, that have not been taken as transfer points are marked with asterisk (*) symbol. The second table M12 named "Acuracy estimation by the differences of transfer point coordinates" characterize the measurements quality of the transfer points. Both tables contain errors in micrometers on photo scale, errors on vertical axis are shown as parallactic. All "dx" columns must always contain values near zero. Mean square deviation values on two other axis may be near the pixel size and maximum deviation values may be near double pixel size. Otherwise, it is necessary to repeat measurements of the transfer points at the conjunction of the corresponding stereopairs. In the worse case you may see the warning "Unable to adjust to scale" at the end of the table M12, which means impossibility to build an arbitrary oriented strip network using existing composition and quality of the measurements on photos.

### 2.5.5. SECTION SM3. Arbitrary oriented strip adjustment

After the preliminary compilation of the arbitrary oriented strip network (section SM2) software starts a joint adjustment of all photogrammetric measurements. Software equates the whole strip the way the set of this common amendment equations will describe a complanarity conditions for all strip points and scale conditions for all transfer points. According to this set of equations a set of united normal equations will be equated. The solution these equations provides us with amendments to the conventional relative orientation elements of the photos. The adjustment results are shown in the section SM3. All values of the tables are in micrometers on the photo scale.

The table M13 "General estimation of the arbitrary oriented strip adjustment accuracy" describes a total number of the amendment equations of different types, a number of accepted and ommited data, mean and mean square, maximum and systematic values of resudual deviations of different types. These values must not be greater than corresponding values from the section SM2. At the bottom of the table the mean square value of the weight unit is shown. It must be near a half of the pixel size for good networks.

The table M14 "Residual errors of arbitrary oriented strip adjustment" will be shown if the software found considerable errors in the input data during the process of compilation and arbitrary oriented strip adjustment and some points are condemned. There are rows containing all points of stereopairs (residual vertical parallax values) and transfer points of the photos (coordinate difference values between adjacent stereopairs) shown consequently in the table. But if the transfer points had been transposed (SECTION SM1), it is nesessary to take into account how the consequence of the strip points have been changed in comparison with the corresponding consequence in the photogrammetric measurements file.

Ommited points are signed with asterisk symbol (*) in the table M14. If the condemnation took place, "The list of points supposed for removing from the strip" table is included to the section. Every row starts with the number of point. Then depending on point type either residual vertical parallax or vertical parallaxes of the previous and next pair and coordinates divergence on the transfer points are shown.

There is a "Delete" sign at the end of the row. You may see this list on the screen as the dialog "The list of points supposed for removing from the strip" during the computational process. Operator may remove the "Delete" sign of some or all points. All the points marked with the "Delete" sign will be shown in the section. It will not affect the data received from Triada.exe. Thus, the operator have to correct these information manually by repeating the measurements on condemned points or by replacing them with new ones.

There is an additional estmantion of the quality of arbitrary oriented strip adjustment in the table M15 "Residual errors of the collinearity conditions of the image points". The maximal errors must not exceed the pixel size and the mean values must be two times as little as the maximal errors  for a good network.

The SM3 section may also contain the table M16 "The list of points with large residual errors of the collinearity conditions". But this list appears if the software unable to condemnate particaular points because of absence of the redundant data or if the operator desided to leave the bad point by removing the "Delete" sign from the corresponding point on the "The list of points supposed for removing from the strip" dialog.

### 2.5.6. SECTION SM4. The general characteristic of the ground-control data

There are the absolute orientation requirements of the strip in this section namely the strip adjustment method, the polynimial type and length for the preliminary (or final) deformation elimination, ground-control points condemnation indication. At the first stage of the strip calculation this requirements are set by default. Thus the section SM4 shows the operator's settings.

The section also contains the information about a number of the strip ground-control points that are subdevided into basic, horizontal and vertical ground-control points.

The information about the presumable accuracy of the projection center coordinates which are defined during the aerophotography using the navigational system (such as GPS) will be included into the section if the corresponding data exists.

### 2.5.7. SECTION SM5. Preliminary strip adjustment by the control data

Preliminary adjustment provides the strip reduction, strip deformation approximation by polynomials and elimination of the manifested deformation. Approximation with the polynomial selection and bad data detection will be applied if the ground-control data composition allows it. The approximate ground-control data error values caused by the preliminary nature of the adjustment. Even in the case of refused data the condemnation validity will be checked lately.

For the existing projection center coordinates the section begins with the table M16 "General estimation of the adjustment of airborne devices data". This table describes a number of accepted and ommited indications on each axis, as well as mean, mean square, maximal and systematic errors. There are error of determination of the enumerated errors and the length of the approximation polynomial at the end of line.

The table M17 "General accuracy estimation of the absolute orientation" with the following type of applied polynomial will appear in the case of existing ground-control points in abundance. There are polynomial length and a number of the ground-control points (accepted as well as rejected) on each axis shown in this table. Then the general data about the divirgence between the photogrammetric and geodetic ground-control point coordinates, as well as the information about the probable calculation accuracy of the network deformation follows. If the block has a great number of ground-control points and their layout allows it, software will change the polynomial length in series with reference to linear, quadric or cubic type. The table M17 contains information about each variant.

The section also contains the table M18 "Residuals of the control points (adjusting by the different polynomials)". It shows coordinate errors obtained during adjustment for each ground-control point particularly. All rejected by the software variants are marked with the asterisk symbol (*) before the axis caption, if the polinomial length has been changed.

### 2.5.8. SECTION SM6. Final results of the rigorous strip adjustment

By default, the final adjustment is carried out using the rigorous method by collinearity conditions. At the same time, all photogrammetric, geodetic and airborne (if they exist) measurments are adjusted jointly in the unified coordinate system. But user may also set polynomial or multiplex method of the final adjustment. The title of the section describes the adjustment method. If some ground-control points have been omitted during the adjustment, corresponding error values are marked with the asterisk symbol.

In the case of existence of the airborne data the section begins with the table M25 "Residual differences of data of the airborne devices". There are serial and actual image numbering and differences of adjusted photogrammetric and airborne values on coordinate axes. There is a generalized description of the airborne data quality in the table M26 "The accuracy estimation of the strip adjustment by the residual differences of differences of the airborne devices".

Similar estimation of the geodetic control data is shown in the table M27 "Residual errors of the ground-control point coordinates" and in the table M28 "The accuracy estimation of the strip adjustment by the ground-control point residual errors".

Analyzing the tables of the SM6 section you have to check the tolerance of the obtained errors.

### 2.5.9. SECTION SM7. Additional testing of the adjusted strip

Additional checking shown in the table M29 "Residual errors of collinearity conditions of the image points" indicates the accordance of all photogrammetric links in the oriented network tolerable to the accuracy of the orbitrary oriented network. This table is similar to the table M15 from the SM3 section. Both tables must have approximately the same degree of the residual error values of the collinearity conditions. If residual errors of a strip part of the M29 table 1,5 times as much as the corresponding data of the table M15, this may stand for an error of the ground-control point which is located on this part of the strip.

In the case of existence of control points which are unused in adjustment the section contains the table M30 "Coordinate deviations on the unused ground-control points". This table shows coordinate deviations on the axes on which this point is defined. There are maximal, mean and systematic coordinate values, as well as a control point number shown below the table.

Mean horizontal differences of the conrol point coordinates must not be more than 0,3 mm. Depending on the vertical interval h mean differences of adjusted heights and geodetic heights of the ground-control points must not be more than 0,2h - 0,35h. Extremly high differences which are equal to double mean difference may occur not often than 10% cases.

Analyzing this table user could make a decision concerning including of some ground-control points to the block. But you have to be sure that software will not condemn this ground-control point because of the rough error on this point.

### 2.5.10. SECTION SM8. Differences of transfer point coordinates of adjacent pairs

This section begins with the table M30 "Differences of coordinates (right pair - left pair)" which shows the corresponding differences for each transfer point of the whole strip. Generalized estimation of separate images are brought together in the table M31 "Accuracy estimation on coordinate differences of transfer points". Values of these two tables must be equivalent to the corresponding values of the tables M13 and M14, if DX and DY values reduce to the image scale and DZ values reduce to parallactic expression. See SM3 section for details.

## 2.6. Strip phototriangulation dialogs

### 2.6.1. Dialog "The list of point supposed for removing from the strip"

Sometimes rough errors occur in photogrammetric measurements. Two cases of this errors appearance during the adjustment of the arbitrary oriented strip are following:

- large residual vertical parallax values on stereopair points;

- large coordinate differences of transfer points calculated on adjacent stereopairs.

Software selects bad points to be removed from the network if this allowed by the network geometry (the case of abundant measurements). In this case you will see the request "Do you want to look through the list of points intended for removal from the strip?". When you select "No" all bad points will be removed from the network and will be unused during the adjustment. Otherwise the dialog containing the foregoing list will appear. Each row of the list containing one point represented by its number, serial numbers of stereopairs containing this point and residual errors on the point. There is a vertical parallax for the points shown only on one stereopair images in the list. There are two vertical parallax values and three coordinate differences on strip axeses for a transfer point shown on three images. All residual errors are in micrometers on the image scale, the heigh differences are interpreted by means of the horizontal parallax error. There is a "Delete" sign in the end of the row. Click on it with the left mouse button to remove it. Click once more to restore "Delete" sign.

All points marked with "Delete" sign will be removed from the strip. Click on "OK" button to apply settings.

### 2.6.2. Dialog "Results of the strip compilation"

This dialog contains sections from SM0 to SM8 of the strip adjusting protocol. Its length and a number of section is changeable depending on the input data composition.

See "Results of the strip compilation" chapter for more informaton.

### 2.6.3. Dialog "Graphic presentation of residuals for the control points"

It is possible to view this dialog in the case of existence of the strip ground-control data and its amount allows to proceed absolute orientation. Use GraphCp button to open this dialog box.

This dialog box contains a strip network scheme as a rectangle and a strip axis with the plane layout of the projection centers marked on it. Compilation control points showed as green triangles, horizontal control points as crimson squares and vertical control points as yellow circles. Control points rejected during adjustment are filled with red color.

There are the following buttons at the top of the dialog box:

Numbers - shows numbers of control points;

dX, m - shows residual values on the axis directed to the North;

dY, m - shows residual values on the axis directed to the East;

dZ, m - shows residual values on the axis directed to zenith;

Vectors dXY - shows horizontal residual vectors relatively to the strip axes;

Vectors dZ - shows vertical residual vectors.

For the conformal geodetic projection with different axis captions the button captions will be changed according to this projection.

Click on a ground-control point symbol to view its number and residual values as geodetic coordinates.

The dialog allows to view the ground-control data used for adjustment, as well as control points which are unused in adjustment. Use "Used" or "Unused" buttons to switch between the corresponding control point groups.

It is possible to set the view limits of the residual differences of the ground-control points making them more representative. Values less than the limit value will not be shown as numbers, as well as vectors. Use "The errors are shown above" radiobutton group to set the limit value. To set the values in this group click on "Errors range correction" button. The "Correction of values of displayed errors" dialog will appear. It is possible to change the size or the scale of the scheme as it mentioned in "Working with the dialog boxes" chapter.

Condemned during adjusting control points are filled with red color. Their captions and vectors have the same color.

### 2.6.4. Dialog "Graphic presentation of differences of transfer point coordinates"

General information about the defferences on transfer points calculated independently on adjacent stereopairs of a strip are shown in the SM8 section of the "Strip compilation and adjustment protocol". This dialog box concretizes the differences showing them graphically as a separate transfer points. Click on "GraphTransP" button to see this dialog.

This dialog box has many similarities with the "Graphic presentation of residuals for the control points" dialog. Both dialogs have the same interface, thus they have similar behavior.

### 2.6.5. Dialog "Correction of values of displayed errors"

The "error" term in this dialog box have common meaning standing for "residual differences", "deviation from the mean value" etc. This dialog appears after strip adjustment, as well as block phototriangulation. Use "Errors range changing" button of other (parent) dialog box to display this dialog.

There is a table on the dialog which allows to set limits for the parent dialog box. Duplicates of table values are shown below and above the table. Fill "Errors in plane" and "Errors in altitude" rows of the table with the corresponding limit values. To select the necessary cell click on it with the mouse or select corresponding radioitem of top or bottom radiogroups. Press Enter key to accept entered value.

### 2.6.6. Dialog "Graphic of collinearity condition residuals"

Use "Photos" button of other dialog to display this dialog box. Depending on adjustment method the dialog title is completed with explanations "arbitrary oriented" or "absolute oriented strip".

There are a photo number and photo scheme with all network points shown on this dialog. There are the following buttons at the top of the dialog box:

Numbers - shows numbers of control points;

dX, micron - shows residual values on the axis directed to the North;

dY, micron - shows residual values on the axis directed to the East;

Vectors dXY - shows horizontal residual vectors relatively to the strip axes.

It is possible to set the view limits of collinarity condition residuals. Values less than the limit value will not be shown as numbers, as well as vectors. Use "Residuals are shown from above, micron" radiogroup to set the limit. It is possible to change the interval between the adjacent radiogroup values using "+1" and "-1" buttons.

Click on a point image to display its number and residual errors on both axes. For the residual values large than double mean square error of the weight unit corresponding values and vectors on the scheme are red.

It is possible to change the dialog box size, as well as scheme zoom. See "Working with the dialog boxes" for details.

Change the "Image number" value to display the corresponding photo of the strip.

### 2.6.7. Dialog "Correction of the requirements for the strip readjustment"

When you receive unsatisfactory strip adjustment results on ground-control data it is advisable to readjust the strip with different conditions. Use "Retry" button for readjusting and this dialog box will appear. In the begining it shows requirements of previosly adjusted strip.

It is possible to change general adjustment requirements by the following ways:

- changing an adjustment type. The most applicable for short strips are polynomial or muliplex methods. For extensive strips or for non-typical ground-control points layout the rigorous method is more advisable. The "Default" value allows the software to choose the method;

- changing a polynom type. Polynimial and multiplex methods require to choose the polynom type which will be used at the final adjustment stage. Thus it is necessary to be attentive choosing the polynom type taking in consideration the geodetic control layout. For the rigorous method polynom is applied for the prior absolute strip orientation and excluding of pronounced strip deformations. Polynoms are not apllied in rigorous adjustment, thus the polynom type is not so important. It is advisable to select "Default" type which allows the software to choose the polynom type automatically;

- setting the polynom power for horizontal and vertical adjusment. For the zero power value software automatically finds an optimal value depending on the ground-control layout;

- setting or resetting "Indication of the horizontal ground points rejection" and "Indication of the vertical ground points rejection" flags.

Click on "Next" button when all settings are done. Use "Back" button to reject readjustment and return to the strip adjustment protocol dialog and attendant graphical dialogs.

### 2.6.8. Dialog "Correction of coordinates and precisions of the control points"

Сlick on "Next" button of the "Correction of the requirements for the strip readjustment" dialog box to switch to this dialog box. There is a "List of the strip ground points (used and not used in adjustment)" table shown in the dialog. It is posible to change using the keyboard the following values:

- horizontal coordinates;

- height;

- mean horizontal error;

- mean vertical error.

Uncheck the "Make correction into RAM only" flag to allow software to correct values in RAM (Random Access Memory) only. Set this flag to save corrections to the block configuration file and to apply changes for the whole block. This will not affect existing files transferred from Triada.exe. You need to make changes to the DPS ground points file by yourself if it necessary.

### 2.6.9. Dialog "Correction of the list of the used or unused ground-control points"

Use "Next" button of the "Correction of coordinates and precisions of the control points" dialog box to bring up this dialog box. There are two tables containing ground-control points which was used (at the left) and unused (at the right) in adjustment process in this dialog box. Click on the cell with the point number to move this point to another table.

Pay attention that correction results are concerned to the current strip. They do not affect block configuration file transferred from Triada. User have to change this file manually if it necessary.

## 2.7. Results of compilation and adjustment of the arbitrary oriented block

### 2.7.1. Overview

As it was mentioned in "Software functions" chapter computation process consists of strip network compilation with the following combining them into the block network using tie points of the overlapping strips and joint adjustment of all photogrammetric data by collinearity conditions.

For the dialog mode the first part of the calculation process displayed as the protocol of compilation and adjustment of the strip phototriangulation.

The second part of the calculation process is stored in FrBlPrt.bbb file as additional text protocol of the arbitrary oriented block phototriangulation adjustment. Protocol information is devided into sections (SECTION SBx), where "x" symbol means a number or a letter.

See chapters below for more information about the contents of this dialog window.

Some tables of these sections show the information about the whole block, but in some cases they are devided into strip parts. In this case every strip part has leading title STRIP IND, where IND is the strip index.

Each section is displayed in a separate page named SB0,... SBC. If the section contains strip tables, additional pages Bx containing correspondnig tables will be created. There are a strip number and the strip index are shown in the corresponding edit boxes.

Futhermore, during the adjustment process the "Tables of differences of tie point coordinates calculated in separate strips" will be stored into the protocol file and shown as a new dialog box containing pages DC1, DC2. They will be created for all combinations of overlapping strips.

Some sections and tables appear in the protocol in the case of existence of corresponding block information. A protocol capacity

depends on the block size, but all standards and tolerances listed in this help are common for any block.

### 2.7.2. SECTION SB0. Common information about strips of the arbitrary oriented block

See Common information about block strips.

### 2.7.3. SECTION SB1. Characteristic of the block geometric scheme

See Characteristic of the block geometric scheme.

### 2.7.4. SECTION SBA. Information about arbitrary oriented block points not accepted as tie points

See Information about block points not accepted as tie points.

### 2.7.5. SECTION SB7. Estimation by the most probably residuals of collinearity conditions

See Estimation by the most probably residuals of collinearity conditions.

### 2.7.6. SECTION SBC. Estimation by coordinate differences of tie points

The arbitrary oriented phototriangulation block is adjusted by collinearity conditions. Only one common for all strips coordinate vector is determined for a terrain point from adjustment independently of the number of strips which the point is present on. But it is important to know the behavior of the point during the following technological processing by separate strips and stereopairs. Thus at the end of adjustment the software will determine coordinate differences of tie points in the transverse overlap zone, if coordinates have been calculated in separate strips range. This estimation is shown in the SBC section. First of all section contains the table B12C "Coordinate differences of tie points calculated for separate strips". It is similar to the table B12A from the SBA section.

### 2.7.7. TABLE DC1. Errors of identification of tie points of adjacent strips

The DC1 table contain probable identification error for each point after transposition of the point from one strip to another. Indexes of the corresponding strip are shown at the top of the table.

Table DC1 and Graphic of errors of identification of tie points on adjacent strips allow to determine the location of a bad identified tie point and make a decision to repeat measurements or to replace this point with the other one.

### 2.7.8. TABLE DC2. General characteristic of errors of identification of tie points

The DC2 table generalizes the data of the DC1 table for each pair of overlapping strips showing mean, mean square, maximal and systematic values of identification errors.

## 2.8. Dialog boxes of the arbitrary oriented block

### 2.8.1. Dialog "Scheme of the arbitrary oriented block"

See Phototriangulation block scheme.

### 2.8.2. Dialog "Results of compilation and  adjustment of the arbitrary oriented block"

See Results of compilation and  adjustment of the phototriangulation block.

### 2.8.3. Dialog "Graphic of residuals of the collinearity conditions for arbitrary oriented block images"

See Graphic of residuals of the collinearity conditions for block images.

### 2.8.4. Dialog "Tables of identification errors of tie points of adjacent strips"

This dialog box contains B1, DC1 and DC2 tables, see Results of compilation and adjustment of the arbitrary oriented block for details. There are three pages on the dialog box. The first page contains B1 table which describes geometric scheme of the block (see Section SB1). The second and third pages contain DC1 and DC2 tables accordingly. There is a tool panel at the top of the dialog box that allows user to display the information about all possible combinations of overlapping strip pairs.

### 2.8.5. Dialog "Graphic of errors of identification of tie points on adjacent strips"

This dialog displays graphically an information from the DC1 table of the results of compilation and adjustment of the arbitrary oriented block. See Table DC1 for details. There is a strip scheme with points which are common for all overlapping strips in the main window, as well as the strip axis with projection centers marked on it. There are the following bottons above the scheme:

Number - shows numbers of points

dX, micron - shows identification error values along the strip;

dY, micron - shows identification error values across the strip;

Vectors dXY, micron - shows identification error vectors relative to the strip axis.

It is possible to set the view limits of identification errors. Values less than the limit value will not be shown as numbers, as well as vectors. Use "Errors are shown from above" radiogroup to set the limit. It is possible to change the interval between the adjacent radiogroup values using "+1" and "-1" buttons.

For the block with transverse overlapping less than 50%, in the case of absence of tie strips, identification errors are shown using the rule: "Adjacent strip minus strip shown on the scheme". Thus equal-by-magnitude, but different by sign errors will be shown for the same point of two neighbour strips. If there are three or more overlapping zones in the block then maximal error values are shown on the scheme for each strip, which are taken for a point of the strip from DC1 tables.

Click on a point image to display its number and identification errors on both axes. For the error values large than triple mean square error of the weight unit, corresponding values and vectors on the scheme are red.

It is possible to change the dialog box size, as well as scheme zoom. See "Working with the dialog boxes" for details.

Specify a strip index or strip number in the corresponding field to display this strip.

## 2.9. Results of compilation and adjustment of the oriented block

### 2.9.1. Overview

As it is mentioned in "Software functions" chapter computation process consists of strip network compilation with the following combining them into the block network using the rigorous adjustment by collinearity conditions taking into account all geodetic and photogrammetric links of the network.

For the dialog mode the first step of the computation process is displayed as the "Results of strip phototriangulation compilation and adjustment". As for the second step of the computation process an additioanal text protocol will be created and stored in OrBIPrt.bbb file. Information of the protocol is combined into sections (SECTION SBx, SRx), where x symbol means particular number or letter.

See chapters below for more information about the contents of this protocol.

Sections contain information as tables, as a rule. Some tables of these sections show the information about th whole block, but some cases they are devided into strip parts. In this case every strip part has leading title STRIP IND, where IND is the strip index.

Each section is displayed in a separate page named SB0,... SB8. If the section contains strip tables, additional pages will be created named Bx and Rx containing correspondnig tables. There are a strip number and strip index are shown in the corresponding edit boxes.

Futhermore, during the adjustment process the "Differences of tie point coordinates calculated in overlapping strips" are shown as a new dialog box containing pages DC3, DC4. This information will be stored into files using the DInd1_Ind2.bbb template, where Ind1 and Ind2 are indexes of two overlapping strips. Software creates them for all combinations of overlapping strips.

Some sections and tables appear in the protocol in the case of existance of corresponding block information. A protocol capacity

depends on block size, but all standards and tolerances listed in this help are common for any block.

### 2.9.2. SECTION SB0. Common information about strips of the oriented block

See Common information about block strips.

### 2.9.3. SECTION SB1. Characteristic of the block geometric scheme

See Characteristic of the block geometric scheme.

### 2.9.4. SECTION SR2. Information about selfcalibration parameters

Considerable part of systematic distortions of the block images are eliminated during measuring process to compile and adjust phtotriangulation by Triada software. This distortions include object-glass distortion, projection rays refraction in atmosphere and photographic film deformation. To eliminate residual influence of the systematic distortions rigorous block adjustment is realized using selfcalibration principle. The resudual distortions are approximated by Dr. Brown's polynoms. During the normal equations solving software analyzes validity of storing of separate coefficients in polynoms and reduces the polynom size to minimize its size. There is a list of coefficients selected by the Photocom software to adjust the block in the section SR2.

Table R2 displays distribution of selfcalibration amendments across the image for point coordinates. Amendments on each axis are shown for tie points. In addition there is dimentions of the image zones which the image devided into shown in this table.

For the phototriangulation block compilated using the data from several cameras an information for each camera will be shown.

It is necessary to estimate quantity of selfcalibration amendments. Their values must not be great than several micrometers, because the influence of considerable part of systematic distortions, as it was mentioned before, has been eliminated by Triada software. Apart from the case of absence of camera object-glass distortion values or they was not taken in account in Triada under some circumstances. In this case depending on the object-glass type and quality selfcalibration amendments at the image edges may achieve several dozen micrometers.

### 2.9.5. SECTION SR3. Control of the oriented block by the complanarity conditions

The final adjustment of the phototriangulation block in Photocom software is carried out by the collinearity conditions. The main aim of the adjustment is to place photos the way all projection rays of identical photo points intersect at the same terrain point exactly. At the same time for the ground-control point photogrammetric coordinates of the intersection point must be identical to geodetic values. If adjustment results meet the collinearity conditions all other photogrammetric requirements will be met automatically including the complanarity conditions. This means absence of vertical parallaxes on stereopair points, as well as absence of coordinate differences of strip tie points calculated independently on adjacent stereopairs. Actually there are measurement errors in a real network and adjusment is realized under condition PVV=min, so it is impossible to achieve absolute fulfilment of all collinearity conditions. So complanarity conditions may be violated measurably.

Section SR3 alows to check satisfaction extent of the complanarity conditions of the oriented block. There is the resulting estimation of residual vertical parallaxes and discrepances of point coordinates in the table R3 of this section. Table R4 shows the distribution of all kinds discrepances in the ranges. All values are dislayed in micrometers in photo scale (horizontal values) and as parallaxes (vertical values). Analyzing this results it is necessary to compare them to pixel size. Each row of the R3 table must contain roughly equivalent number of positive and negative values and systematic error must be about zero. A number of discrepances in the range must quickly decrease as its limits increase. For a part of the table describing links inside strips an upper limit of the third range is accepted equal to pixel size. An upper limit of the 5th range is equal to double pixel size and an upper limit of the 6th range is equal to triple pixel size. Other limits are interpolated using indicated values. For a part of the table describing links between strips, limits of ranges are doubled.

### 2.9.6. SECTION SBA. Information about rigorous oriented block points not accepted as tie points

See Information about block points not accepted as tie points.

### 2.9.7. SECTION SB3. Information about appliction of airborne devices

For the block or its part that has projection center coordinates and "PhotoCom|Use projection centers" flag of the Triada sotfware is set then the SB3 section will be added to the protocol. First of all this section contains table B9 "Residuals of the airborne devices data". There are horizontal and vertical descrepancies of all projection centers which are provided with airborne devices data during the aerophotography for each strip shown in this table. The descrepancies are computed as differences between photogrammetric center coordinates and adjusted airborne values. Adjusted airborne values are the airborne data without systematic errors. Validity of these errors determination depends on geodetic ground data scheme, i.e. on the number of ground points and their location.

Table B10 "Estimation of the block accuracy by the resuduals of airborn devices data" describes the data of the B9 table in details. There are mean, mean square, systematic and maximal horizontal and vertical errors for each strip shown in this table. An additional table B11 characterizes interval distribution of deviations.

Tables B10 and B11 are shown in the SB3 section of the "Results of compilation and adjustment of the phototriangulation block" dialog box. Table B9 is displayed as separate strip parts in an individual page with the same caption.

### 2.9.8. SECTION SB5. Estimation by ground-control points participating in adjustment

If phototriangulation block provided with ground-control data and points participating in adjustment specified in the network configuration file, protocol contains section SB5. It consists of two tables.

The table B15 displays the summary block accuracy estimation by residual errors of the ground-control points. Information displayed for the strips containing ground-control points and consists of mean, mean square, maximal and systematic error obtained using the data of the strip tables B17 described below. The table B16 shows interval distribution of deviations for the whole block. Generally this two tables are similar to B10 and B11 tables of the SB3 section.

Table B17 "Deviations of ground-control point coordinates used in adjustment" describes the strip part of the SB5 section. There are deviations obtained using the template "adjusted photogrammetric coordinates minus their geodetic values" for each ground-control point displayed by turns in this table. Naturaly deviation values of the ground-control point contained by two or more strips are equal for all strips. Coordinate deviations on the condemned during adjustment ground point marked with the asterisk symbol.

Tables B15 and B16 are shown in the SB5 section of the "Results of compilation and adjustment of the phototriangulation block" dialog box. Table B17 is displayed as separate strip parts in an individual page with the same caption.

### 2.9.9. SECTION SB6. Accuracy estimation by ground-control points unused in adjustment

For the control points unused in adjustment protocol contains section SB6. First of all it contains B18 table devided into parts as separate strips. There are residual errors of ground-control points unused in adjustement in this parts. If there is not control points in the strip the B18 table does not contain information about this strip.

Table B19 discribes the summary block accuracy estimation by residual errors of ground-control points. There are mean, mean square, maximal and systematic errors for the strip with the control points obtained unsing template "adjusted photogrammetric coordinate minus geodetic coordinate" in this table. Additional table B20 characterizes range error distribution. Generally these two tables are similar to B15 and B16 tables of the SB5 section.

Tables B19 and B20 are shown in the SB6 page of the "Results of compilation and adjustment of the phototriangulation block" dialog box. Table B18 is displayed as separate strip parts in an individual page with the same caption.

### 2.9.10. SECTION SB9. Lists of deviations of block ground-control point coordinates

This section replicates data of B17 (see SECTION SB5) and B18 (see SECTION SB6) tables. But all information about deviations are displayed for the whole block. Table BOP shows the "Block list of residual errors for ground-control points used in adjustment". Each ground point appears once in the list. Condemned coordinate deviations marked with the asterisk symbol. Table BCP contains the similar "List of residual errors for ground-control points not used in adjustement".

### 2.9.11. SECTION SB7. Accuracy estimation of rigorous adjusted block by residuals of the collinearity conditions

See Estimation by the most probable residuals of the collinearity conditions.

### 2.9.12. SECTION SR5. Probable accuracy of the adjusted block

When in dialog mode aside from computation of adjusted unknown values software estimates determination accuracy of this unknown and first of all adjusted terrain point coordinates will be estimated. Matrix of coefficients of normal equations is inverted for this purposes, then weight functions of terrain point coordinates are determined. Using this functions and a mean square error of the weight unit software determines the most probable coordinate accuracy. Probable accuracy data is generalized for stereopairs and strips and then software puts them into SR5 section.

Estimation results concerning strips are displayed in R8 table "Distribution of accuracy of the terrain point coordinates by stereopairs". There is a stereopair number in the strip, the number of points in the stereopair, as well as minimal, mean and maximal error on points of this pair on three coordinate axes. The last line "for strip" summarizes data of all stereopairs.

Block estimation data is shown in the R9 table "Distribution of accuracy of the terrain point coordinates by strips". This table is assembled from the last lines of separate strips. There are strip index, a number of terrain points in the strip and probalbe errors on three axes (minimal, mean and maximal).

Analyzing data of the SR5 section it is necessary to determine proportional accuracy of adjusted terrain point coordinates of all strips and stereopairs. Obtained coordinate accuracy must correspond mapping task requirements or tolerable for the spatial phototriangulation purposes.

Table R9 is shown in the SR5 page of the "Results of compilation and adjustment of the phototriangulation block" dialog box. Table R8 is displayed as separate strip parts in an individual page with the same caption.

### 2.9.13. SECTION SB8. List of adjusted block strips

During the adjustment process some strips of the block may not be ready to be included into the block. As a rule it caused by the presence of the great number of gross errors in the input data. In this case Photocom software excludes bad strips and informs user about this and depending on the user agreement make an attempt to build and adjust reduced block. Information about the reason of excluding of strips is stored into protocol. As a rule this information is at the beginning of the protocol, but could be at any position. Section SB8 displays the list of adjusted block strips. If there are all strips in the list, it means that the computation process formally finished completely.

### 2.9.14. TABLE DC3. Coordinate differences of tie points calculated in overlapping stris

Rigorous adjustment of the oriented phototriangulation block is realized by the collinearity conditions. There is only one common for all strips coordinate vector value independently of the number of strips a terrain point present at. But it is important to know the behavour of this point at the following technological proceeding by separate strips and stereopairs. Thus at the end of adjustment software determines values of coordinate differences of tie points which are in vertical overlap zone for each strip separately. There is the strip index at the top of the table.

It is possible to estimate using the DC3 table with great likelihood how contours and contourlines will be joined at bounds of neighbour strips during collection of digital and graphical maps, orthophotos etc. This table with "Graphic presentation of tie point coorditate differences" dialog box allows to determine the position of a badly identified tie point and make a decision concerning remeasuring it or replacing this point with the other one.

### 2.9.15. TABLE DC4. General data on tie point coordinate differences of strips

Table DC4 summarizes the DC3 table data for each pair of overlapping strips displaying mean, mean square, maximal and systematic difference of tie point coordinates on three axes.

## 2.10. Dialog boxes of the geodetic oriented block

### 2.10.1. Dialog "Scheme of geodetic oriented block"

See "Scheme of the phototriangulation block"

### 2.10.2. Dialog "Results of compilation and adjustment of the oriented block"

See "Results of compilation and adjustment of the phototriangulation block"

### 2.10.3. Dialog "Graphic representation of residual errors of block ground-control points"

This dialog displays the information of the text protocol of the SB9 section graphically. Click on BlockGraph button in the dialog containing this button to bring on this dialog box. There are bounds of all strips in this dialog box and all existing ground-control points. Each strip contour has different color, thus it is easy to identify their overlap zones. This dialog is similar to dialog "Graphic presentation of differences of transfer point coordinates".

### 2.10.4. Dialog "Graphic presentation of residual errors of ground-control points"

This dialog displays the information of the text protocol of the B17 table (SB5 section) and B18 table (SB6 section) graphically. Click on StripGraph button in the dialog containing this button to bring on this dialog box. This dialog box is similar to "Graphic presentation of residuals for the control points" dialog. In addition it is possible to select necessary strip specifying its number or index.

### 2.10.5. Dialog "Graphic of collinearity conditions residuals (geodetically oriented block)"

See "Graphic of collinearity conditions residuals for block images"

### 2.10.6. Dialog "Graphic presentation of terrain point coordinates accuracy"

This dialog displays the information of the text protocol of the SR5 section graphically. Click on GraphAccur button in the dialog containing this button to bring on this dialog box. There is a graphic image of terrain point coordinates accuracy in this dialog box.

There is a strip contour with all terrain points inside the strip in the dialog box. In addition strip axis and projection centers are shown. Click with the right mouse button on image center to display a number of this image.

There are the following buttons at the top of the dialog box:

Numbers - shows numbers of control point;

dX, m - shows residual values on the axis directed to the North;

dY, m - shows residual values on the axis directed to the East;

dZ, m - shows residual values on the axis directed to zenith;

Vectors dXY - shows horizontal residual vectors relatively to the strip axes;

Vectors dZ - shows vertical residual vectors.

For the conformal geodetic projection with different axis captions the button captions will be changed according to this projection.

Click on a ground-control point symbol to view its number and its most probable residual values.

It is possible to set the view limits of the probable residual errors making them more representative. Values less than the limit value will not be shown as numbers, as well as vectors. Use "The errors are shown from above" radiobutton group to set the limit value. To set the values in this group click on "Errors range correction" button. The "Correction of values of displayed errors" dialog will appear. It is possible to change the size or the scale of the scheme as it was mentioned in "Working with the dialog boxes" chapter.

Specify a strip number or its index to switch to another strip.

### 2.10.7. Dialog "Differences of tie point coordinates calculated in overlapping strips"

This dialog displays DC3 and DC4 tables from DInd1_Ind2.bbb files. These tables contain additional information to "Protocol of compilation and adjustment of the oriented block". Thare are three pages on this dialog box. The first page contains B1 table which characterizes geometric scheme of the phototriangulation block. The second and the third page contain DC3 and DC4 tables accordingly. There are tool panel at the top of the dialog box allowing to display all possibe pair combinations of overlapping strips.

Click on DifText button in the dialog containing this button to bring on this dialog box.

### 2.10.8. Dialog "Graphic presentation of tie point coorditate differences"

Click on DifGraph button in the dialog containing this button to bring on this dialog box. This dialog box is useful for graphical interpretation of the information containing in the DC3 table "Coordinate differences of tie points calculated in overlapping stris" and in DInd1_Ind2.bbb file which are additional to "Protocol of compilation and adjustment of the oriented block". There is a strip scheme with points lying in the overlap zone with an adjacent strip which has a greater number shown in this dialog box. Indexes and numbers of both strips are displayed on the top toolbox. All scheme information correspond to the particular strip pair. There is a main strip axis with projection centers marked on it. It is possible to create a caption of a strip number clicking with the right mouse button on its center.

There are the following buttons above the scheme:

Numbers - shows numbers of control points;

dX, m - shows residual values on the axis directed to the North;

dY, m - shows residual values on the axis directed to the East;

dZ, m - shows residual values on the axis directed to zenith;

Vectors dXY - shows horizontal residual vectors relatively to the strip axes;

Vectors dZ - shows vertical residual vectors.

For the conformal geodetic projection with different axis captions the button captions will be changed according to this projection.

Click with the left mouse button on a point symbol to view its number and its coordinate differences.

It is possible to set the view limits of the coordinate differences making them more representative. Values less than the limit value will not be shown as numbers, as well as vectors. Use "The differences are shown from above" radiobutton group to set the limit value. To set the values in this group click on "Differences range correction" button. The "Correction of values of displayed errors" dialog will appear. It is possible to change the size or the scale of the scheme as it mentioned in "Working with the dialog boxes" chapter.

Specify a strip number or its index to switch to another strip.

## 2.11. Common information in protocols of arbitrary and rigorous adjusted block

### 2.11.1. Common strip information

Section SB0 begins the block adjustment protocol. There are the list of strips proposed for adjustment in this section. For each strip it displays strip index, the number of images, as well as ground and ground-control points of all images.

For arbitrary block adjustment one strip marked as "Main". The rest strips of the phototriangulation block are oriented according to coordinate system of this strip. As a rule the software sets the longest strip as the main.

### 2.11.2. Characteristic of the block geometric scheme

A block scheme is characterized in SB1 section using a data about the location of tie points on overlapping strips. Depending on the block scheme table B1 "Number of tie points for strips" shown as a band or as a triangle. Strip indexes are listed in the table in the left vertical column and in the top horizontal row. The main diagonal for each strip displays the number of points also shown on other strips. There is the number of tie points for strip pairs which indexes are shown in the corresponding rows and columns at the right to the main diagonal at an intersection of rows and columns.

Table B1 may not fit to standard page by width for a large block. In this case it will be devided into several parts.

### 2.11.3. Information about block points not accepted as tie points

This section consists of table B12A "List of coordinate differences of points not accepted as tie points". The list is relative to ground points. There is a little separate table for each point containing its number, indexes of strips to which this point belongs and coordinate abmodality for separate strips. If the point belong to two strips, it is naturally that absolute values of abmodality for both strips are equal, but have different sign.

SBA section does not appear for a good block.

### 2.11.4. Estimation of accuracy of adjustment by residuals of the collinearity conditions

SB7 section contains the most important information about block adjustment accuracy. All estimation values are obtained using probable amendments of measured coordinates of image points. In turn the most probable amendments mean residual errors of collinearity conditions with reversed signs.

Table B21 describes information for separate strips with their indexes shown before informal part of the table. Structure and contents of the table is similar to table M15 (see SM3 section) and table M29 (see SM7 section) of the protocol of compilation and adjustment of strip phototriangulation. The summary row "Mean for the strip" is displayed for each strip in the table except rows for separate images. The data in summary rows for all strips must be the same degree.

If the most probable residuals of some strip points greater than the doubled pixel size, they will be bring out into the table B22. This table is divided into the parts of single strips. Each row contains the information about two or one point. The information includes a number of strip image (serial and real), a number of the point and its most probable residuals along image axis. This table with the graphic of residuals of the collinearity conditions for block images allows to determine bad points and to make decision concerning the correction of measurements or replacement of this points with other ones.

If there is no any point with the large most probable residuals of collinearity condition, it's noted below the strip index in the protocol that there is no data for it. In the good block the table B22 can be completely absent. The table B23 begins the protocol page, which belong to the whole block. It's headed with "Estimation of accuracy of adjustment by the most probable residuals of the collinearity conditions". This estimation is given separately along the image axis. It includes mean, mean square, systematic and maximal residual and the total quantity of residuals with their subdivision into the positive and negative values. Actually the total estimation generalizes the data of all final rows

"Strip average" from the table B21 wich belongs to the same section decribed above. In the good block mean values of the most probable residuals must not be greater corresponding values from the table M15 of the strip phototriangulation protocol (see SM3 section) more then 20-30%. The values of the maximal error can be under sesquialteral value of pixel. It's necessary to be sure that systematic component of the most probable amendments are about zero, but the quantity of positive and negative residuals is almost the same.

Distribution of the most  probable residuals of collinearity conditions in the ranges is shown in the table B24. Main boundary of the third interval is regarded as equal pixel size. Main boundary of the fifth interval is equal double, and the sixth interval is equal to the triple pixel size. Other boundaries of intervals are interpolated from the specified values. It is necessary to be convinced, that with increase of interval boundaries the quantity of residuals which are inside it quickly decreases, corresponding to the law of normal distribution of errors of measurements.

At last, table B25 "Distribution of colinearity conditions residuals by strips" allows to estimate how the values of residuals change on the area of the block. It consists of final rows "Strip average" taken from the table B21. In the good block all strips should have identical parameters of accuracy.

The value of the mean square error of a weight unit closes section SB7. It should not be close to half of pixel size.

The tables B23, B24, B25 and the mean square error of a weight unit are placed to section SB7. Tables B21 and B22 are displayed as parts of separate strips on individual pages with the same captions.

## 2.12. Common dialog boxes of arbitrary and rigorous adjusted block

### 2.12.1. Phototriangulation block scheme

Bundle adjustment always begins from this passive dialog box. If you want to bring on this dialog box during the construction of an arbitrary oriented network in another way then click on BlockGraph in the dialog containing this button.

The dialog displays the block scheme the way their overlapping zones are notable. There are tie points which connect strips in an united network are shown on the scheme. During the oriented block construction the ground points are put on the scheme.

During the construction of an orbitrary oriented block if it is allowed by the structure of the block ground data to define direction angle of the main strip, strips on the scheme are correctly oriented relatively to the terrain coordinate system. Otherwise the axis of the basic strip is oriented from left to right on the scheme.

### 2.12.2. Results of compilation and  adjustment of the phototriangulation block

Click on BlockText button in the dialog containing this button to bring on this dialog box. The dialog displays a part of the report of adjustment of the arbitrary or geodetic oriented phototriangulation block. This part concerns to sections SBx and SRx, where x is any letter or figure. The following chapters describe the structure of the report of adjustment of particular type:

- Results of compilation and adjustment of the arbitrary oriented block

- Results of compilation and adjustment of the geodetic oriented block

Each section of the report is displayed on the page which have corresponding caption. Bx or Rx captions of pages contain tables for separate strips. It is possible to display the information about any strip of the block specifying a serial number or an index of the necessary strip in the corresponding object at the top of the dialog box.

### 2.12.3. Graphic of residuals of the collinearity conditions for block images

Click on Photos button in the dialog containing this button to bring on this dialog box. This dialog contains all same objects as a strip phototriangulation (see Dialog "Graphic of collinearity conditions residuals"). But there is an additional panel on the dialog for the block. It is possible to display necessary images specifying their number or an index.

# 3. Creating of the geodetic projection library

## 3.1. Main information about software

### 3.1.1. Application

Software allows you to create and edit geodetic projections. This information will be used by adjusting software Photocom.exe.

ConfProj.exe must be placed in the same folder with Photocom.exe.

### 3.1.2. Conformal geodetic projection library

Conformal geodetic projection library is a file which contains information about projections in certain order. Its name is ProjFile and it is created at the first software execution. Software places it into folder Library, which must be created in the same folder as the main software before running ConfProj.exe.

Software allows to work with three most spread projections: transverse cylindrical, conical, stereographic. Projection properties, such as Earth ellipsoid dimensions, designations of the coordinate axes, coordinate system origin, projection scale and others are not limited, but they must correspond to real values. There is no restriction for the projection file size. It is enough to store information about one projection, which is useful for day-to-day activity. If there is no projections available the transverse cylindrical Gauss-Kruger projection on the Krasovsky reference ellipsoid accepted in the former USSR for coordinate systems of 1942 and 1995 year.

ProjFile contains binary data and could be modified using ConfProj.exe only.

### 3.1.3. Functions

СonfProj has the following functions:

- creating conformal geodetic projection library;

- input and control of the input data;

- forming of a structure that describes a newly created projection, comparing this structure with the library contents and saving it into the projection library file if it necessary;

- showing projection information on the screen and storing this information into the text file;

- erasing unnecessary projection;

- setting an active projection for using in Photocom.exe.

### 3.1.4. Interaction between projection libaray and adjusting software

If none projection from the projection list selected, software takes the first projection from the list as the active projection. If ProjFile does not exist software uses transverse cylindrical Gauss-Kruger projection on the Krasovsky reference ellipsoid for coordinates transforming.

Accuracy of transformation of coordinates on the zone edge is less than 1 mm.

## 3.2. Using software

### 3.2.1. Creating a new geodetic projection

Program ConfProj.exe creates a new projection under the following circumstances:

- if the projection library is not yet created or there is no any projection in it;

- if operator has chosen item "to prepare a new projection" in the main menu of the software of the dialog

"Type of operations with the topographic map projections".

Beginning to prepeare a new projection, operator should represent precisely the following:

- the projection type and ellipsoid to create;

- designations of coordinate axes;

- the location of the point of origin;

- coordinate values of the initial point;

- the projection scale for the central meridian;

- and, at last, the zone served by the projection.

The structure of dialogs boxes of the software and a sequence of their representation on the screen depends on type of a projection. Choose the necessary value from the drop-down list or enter the necessary value.

The program analyzes entered values, comparing it with possible values. If the admission is exceeded error message will appear. For example, if you specify latitude of a standard parallel equal to 89° for the conic projection the program will inform, that the projection won't be sold for equatorial and subpolar zones. In some cases receiving the confirmation software accepts the doubtful value. However if values several times larger than the limit values calculation process will be interrupted with the indication of the reason for it, for example, because of the abnormal dimentions of ellipsoid.

When operator finishes inputing data software will display the full list of the entered parameters of a projection and offer the operator to prove it. Having received confirmation, the program forms structure with the description of a created projection and compares this structure to contents of the library. If the same structure in library is not present, file ProjFile is updated and the new structure is added. Otherwise the message "The necessary projection is already present in library" will appear.

Use "To mark the projection as default used" checkbox to set the default projection.

### 3.2.2. Viewing the projections library

Choose "To look through the library of projections" item from the "Type of operations with the topographic map projections" dialog to see parameters of all projections from library by turns. Use OK button to switch to the next projection. To interrupt viewing click on Cancel button.

Displaying information about projections software will log this information into protocol if the appropriate item of the main menu is chosen. The protocol is stored in ...\Library\Protproj.prt file.

### 3.2.3. Deleting the projection from the library

It is possible to remove unnecessary projection from the library. All projections in the library are numbered. The software asks to specify a number of projection for removal in the appropriate dialog. Before removal the program displays the information about the projection. Be sure that you have chosen right projection and click on OK button to delete it.

There is no functionality to undo projection deleting. Thus the numbers of all other projections placed in the library behind the removed projection, will be decreased by one.

### 3.2.4. Setting the default projection

If the library consists of one projection only this projection will be chosen for transformation of the coordinates. In other case you need to select the projection as default for further using in Photocom software. It is possible to set the default projection both at its creation, and at activization of item "to mark a projection" of the "View of the operations which are perfomed by library" dialog.

Set the "To mark the projection as default used" checkbox at the bottom of the dialog "Information about projection" to make this projection default. The last line of the projection information labeled as default contains corresponding note.

The easiest way to select the necessary projection is to set the appropriate checkbox for the necessary projection looking through the projection library.

## 3.3. Dialog boxes

### 3.3.1. Dialog "Type of operations with the topographic map projections"

This dialog box allows to do the following operations:

- to prepare a new projection

- to look through the projection library

- to output the the information about projections into a protocol

- to remove a projection from library

- to mark a projection for use as default

Select the necessary command and click on OK button.

If the case of absence of the projection library (i.e. file Projfile does not exist) the software will follow the "to prepare a new projection" command instead of displaying this dialog box.

### 3.3.2. Dialog "A topographic map projection and an earth ellipsoid"

This dialog contains two radiogroups. The Projection radiogroup allows to set one of the following kinds of the conformal geodetic projections: transverse, normal conic or stereographic. The Earth ellipsoid radiogroup allows either to select one of seven earth ellipsoids with the standard parameters or special ellipsoid. The last case needs the dimensions of earth ellipsoid to be specified in the "Dimensions of the special earth ellipsoid" dialog.

### 3.3.3. Dialog "Dimensions of the special earth ellipsoid"

Special ellipsoid is an ellipsoid which isn't one of seven ellipsoids listed in the "Geodetic projection and special earth ellipsoid" dialog. User must specify the following values for the ellipsoid:

- The major semi-axis;

- The minor semi-axis;

- The flattening denominator;

- The first eccentricity;

- The second eccentricity.

Some values of the dialog has an exact mathematical dependence, so it is enough to enter only two values, but even one of them must be linear. Specified values should correspond to the real form of the Earth.

### 3.3.4. Dialog "Designations of the coordinate axes"

This dialog box allows to specify corresponding designations for the coordinate axes. These designations will be used by Photocom software for displaying information, such as protocols, tables and reports. Software always changes the letter cases to upper.

### 3.3.5. Dialog "The coordinate system origin and the projection scale"

In this dialog box user have to select from the drop-down list or specify manually the following values:

- Coordinate of points on the equator line, i.e. the value of the point of origin on the north-directed axis. In the Gauss-Kruger projection, which is used in Russia, this value is equal to zero. In some Mercator projections, if serviced area including both northern hemisphere and southern hemisphere, it equals to 10000000 meters;

- Coordinate of points on central meridian, i.e. the value of the point of origin on the east-directed axis. For Russian coordinate systems of 1942 and 1995 years this value equals to 500 000 meters;

- Projection scale for the central meridian. For Gauss projection it equals to 1.0000, for UTM projection this value equals to 0,9996.

### 3.3.6. Dialog "The number of grid zones"

Select the number of zones (one, two or three) for the conic projection. If your country uses more than three coordinate zones it is necessary to include the set of projections containing three zones to the library. For example, the first projection for zones numbered as 1, 2 and 3 and  the second projection for zones numbered as 2, 3 and 4. This variant allows to serve the territory of the whole country and all overlap areas of coordinate zones.

### 3.3.7. Dialog "Denomintaion of the grid coordinate zones"

Confirm the request "Are there specific names of the coordinates grid zones (Push Y/N)" by clicking on Yes button to display this dialog box. You have to enter denomination of zones in this dialog. Depending on the number of zones there are one, two or three lines appeared in the dialog. The length of denomination is limited by the text field size.

### 3.3.8. Dialog "Position of the coordinate system"

This dialog is used for a concrete definition of the territory served by a conical projection. Use coordinate system data which is officially accepted in the country situated within borders of territory served by the projection. For the conic projection defined by two standard parallels which cone the earth ellipsoid equivalent latitude must be calculated using latitudes of these parallels. Specify the equivalent latitude value in this dialog. If you have one standard parallel, i.e. a cone touching the ellipsoid, instead of equivalent latitude enter the standard latitude value.

All angular sizes are in degrees with minutes and seconds if they available. The separator between degrees and minutes, as well as between minutes and seconds is a space symbol. Seconds may have a fractional part. The decimal separator is a "." (point) symbol. Linear values must be in meters with the number of significant figures after a decimal point according to the projection. For the touch cone the projection scale may equal to 1.0 and for the secant cone the projection scale must be from 0.999 to 1.0.

As the result of entering data the following values must be specified:

Latitude of the equivalent (standard) parallel, deg. min. sec.;

Longitude of the axial meridian, deg. min. sec. (within the range: 0 to ±180 degrees);

Borders of served territory: northern, southern, east, western;

Scale of a projection on an equivalent (standard) parallel;

Coordinate of the east-directed axis;

Coordinate of the axial meridian of a zone.

### 3.3.9. Dialog "Position and parameters of the stereographic projection"

This dialog concretizes data for the stereographic projection. Use coordinate system data which is officially accepted in the country situated within borders of territory served by the projection.

You have to enter the following values in this dialog:

Latitude of the system coordinate origin Bo;

Longitude of the system coordinate origin Lo;

Radius of the zone served by the projection;

Constant coordinates of points on the north-directed axis;

Constant coordinates of points on the east-directed axis;

Projection scale at the system coordinate origin.

Constant coordinates of points on axes mean coordinate values of the point of origin. The separator between degrees and minutes, as well as betweem minutes and seconds is a space symbol. Seconds may have a fractional part. The decimal separator is a "." (point) symbol. Linear values must be in meters with the number of significant figures after a decimal point according to the projection. For the touch cone the projection scale may equal to 1.0 and for the secant cone the projection scale must be from 0.999 to 1.0.

### 3.3.10. Dialog "Information about the projection"

This dialog displays the information about the projections. It consists of projection type, ellipsoid parameters, the designations of coordinate axes, coordinates of the point of origin and projection scale. For a conic and stereographic projection the latitude and the longitude of the point of origin, borders of served territory are also shown.

Each projection in library has its own number. It is shown at the dialog title.

This dialog is displayed for all operations carried out by the library. However the dialog title may vary according to particular operation.

After creation of a new projection it is necessary to look through the data in the dialog and confirm the correctness of data by clicking on OK button. Receiving the confirmation software saves the projection data to the file \Library\ProjFile.