The instruction for adjustment of stereoscope with a monitor

There are two methods of a 3D stereo: stereo shutter-glasses and stereoscope. Regardless of the fact that the stereoscope distorts stereoimage more than stereoglasses most of operators prefer to work with this device. The eyes and central nervous system do not get so tired.

To provide comfortable work the stereoscope has wide ocular lens. But a stereoimage becomes deformed because of completely unavoidable spherical aberration. It also causes convergence or divergence of eye directions of operators, whose eye base is far out of 64 mm.

The stereoscope is equipped by optical wedges to avoid non-parallelism of operator’s eye directions.

The image curvature value can be great if the optical wedges are not adjusted properly.

If sufficient perception is provided without wedges, you can remove them from the stereoscope or fit marks on the movable and stationary blocks of the nozzles. In this case movable and stationary block form a parallel-sided plate, which does not cause distortion of an image. The marks of stationary blocks should be located above if operator has no squint. Mutual displacement of the movable and stationary blocks should be almost equal.

There is no the sharpness adjustment in the stereoscope. You can adjust it by changing the distance between monitor and the stereoscope (by moving of monitor) in case of operator’s myopia. It causes decreasing of field of vision, but provides sufficient perception.

The operator’s workstation should be mounted as shown on a given scheme (fig.1) where you can find average values of adjustable dimensions.

Perform the following operations for mounting of operator’s workstation:

1. Attach stands of the handwheels to an operator’s table at a distance ~65 mm from a table front edge.

The height of hand wheels mounting must be the most suitable for operator.

2. Adjust chair height to suitable for operator position. It must provide the convenient work with the footdisk and the pedals.

3. Set the operator’s chair to operate with hand wheels.

4. Put the stereoscope holder on the table surface at a distance ~ 315 mm from a table front edge to a holder leading edge.

5. Remove the nozzle with wedges from the stereoscope and attach the stereoscope to the holder.

6. Fix the stereoscope eye-glasses on suitable for an operator height (730-820 mm from a chair to eye pupils). Holder’s position must be fixed by a support in stands groove.

7. Moving a stand with the stereoscope try to place the stereoscope eye-glasses more suitable for an operator in horizontal plane relative to hand wheels and chair.

8. Place monitor on the stand and set distance from monitor to the stereoscope input lenses in range of 600 mm.

9. Lift up the stereoscope with its holder not to prevent to adjust parameters of the monitor.

10. Switch on computer and monitor. Open a test object on the monitor display (program Grid.exe in \Delta\Utils folder).

11. All sides of inner squares of left and right fields should be equal to 170±0,5 mm.  For that you must adjust image extension on the screen. Measure all dimensions directly on a screen using a measuring ruler.

12. Fit outer contours of left and right grid with corresponding fields of view of the stereoscope by moving the monitor and the stereoscope on a holder.

13. The monitor screen and optical axes of input lenses should be mutual perpendicular. Incline the monitor screen and stereoscope holder for that.

14. Attach nozzles with wedges to the stereoscope and superpose the marks on movable and stationary blocks with each other so that marks are located above (see fig.2). Use lock screws to fix stationary blocks of the nozzles.

15. The sufficient perception and the minimal curvature of a stereoimage can be adjusted by rotation of the movable blocks of the nozzles (see fig.2). Perform wedge adjustment to make the stereoimage as plane as possible.

16. If stereoimage is insufficiently planed you should also unlock stationary blocks. Gradually rotate stationary blocks (and movable block as it is described in 15th item) relative to nominal position.

17. Correct extension of the image on the monitor screen to provide the flat stereoimage.

18. To provide the sufficient perception of the stereoimage the distance from the stereoscope to the monitor can be decreased.